How to treat natural stone.

Polished surface indoor: You do not need to have anything on it.

Outdoor on sandbrushed and honed surface:  Once a year, put on  impregnation for stone. This seals pores and maintain colors. This is good as protecting from acid rain, and organic plants.

Outdoor, it is best to place stones in sunny places.

Natural stones is best to maintenance with only water. All soap must be natural. After, wipe with a dry cloth.

Some things that should be kept away from stones: Fiber clothe, acid as in citrus fruit, alcohol, chemical soap.

Assembly of kithentops.

The slabs with holes for sink and stove is  packed with  wood all around. This wood must not be taken away before the  slab has been put on place.

The slab must be carried standing.

If putting together to slabs, use some thick paper inbetween. After when they are almost together, and in same direction, take away the papir.

As  glue, Tec7 is  good.

By measurement:

If there is going to be a sink or stove in the slab, it is best to have as much possible stone in front and back. The slabs widness as much as possible. For sinks it is good to have them underplaced. This gives a smaller hole.

To place the hole, give the lenght to center hole, from the wall or cabinet.

If the stone end out in free at lenght, make the outside measurement together with total lenght. The outside meaurement is normaly 15 mm.

Mark with a X the frontside that are to be polished.

It should be one or two mm lesser size if slabs  have wall in both sides. Pls tell me if this have been considered. Lots of information is good.

Værlandet, 22 oktober 2018

Laila Landøy, mobile +47 90826242. mail: laila@landoy.no